Office and User Connectivity Support

| Process Owner | Endpoint Management / IT Operations | Approver | Manager / Service Owner |
| --- | --- | --- | --- |
| Prepared By | Dnyaneshwar Karande | Version | 1.0 |
| Classification | Internal | Status | Handover Draft |

# 1. Purpose and Scope

To diagnose Wi-Fi, wired internet, VPN and office connectivity issues using a structured process and escalate with useful evidence.

| Trigger | User cannot connect, slow/unstable network, VPN failure, office outage, meeting disruption, or new desk setup. |
| --- | --- |
| Prerequisites | User/device details; location; affected service; network access; authorized diagnostic tools; ISP/network owner contacts; known maintenance/outage information. |
| Expected output | Restored connectivity or complete escalation containing scope, diagnostics, timestamps and workaround. |
| General control | Use approved request, least-privilege admin role, masked evidence, and user/owner confirmation. |

# 2. Detailed Procedure

## Step 1: Capture scope and impact

Record location, device, connection type, time started, exact error and affected applications.

Check whether one user, one floor/site, or all users are affected.

Checkpoint: Incident priority and ownership are clear.

## Step 2: Check physical and local basics

Check power, cables, dock, Ethernet link light, Wi-Fi enabled, airplane mode and correct SSID.

Restart adapter/device only when business impact permits.

Checkpoint: Physical/local issue is ruled out or fixed.

## Step 3: Compare connection paths

Test wired versus Wi-Fi, approved hotspot, another device and browser/app where possible.

For VPN, test internet before VPN.

Checkpoint: Failure is isolated to device, network, VPN or service.

## Step 4: Capture network diagnostics

Record IP configuration, gateway reachability, DNS behavior, latency/packet loss and VPN client status using approved tools.

Capture exact timestamps and destination/service.

Checkpoint: Escalation evidence is meaningful.

## Step 5: Check account, certificate and client

For VPN, confirm user access, MFA, certificate, device compliance, client version and gateway selection.

Do not bypass security or install unapproved VPN clients.

Checkpoint: Authentication/client cause is confirmed or ruled out.

## Step 6: Apply safe resolution

Reconnect network, renew approved settings, update supported network/VPN client, correct adapter selection or move to stable connection.

For Cloud PC/meeting, prefer stable Wi-Fi or wired connection.

Checkpoint: User can repeat the failed task.

## Step 7: Escalate office/ISP issue

Provide affected count/location, circuit/reference, start time, diagnostics, device comparison and business impact.

Track case number, ETA and updates.

Checkpoint: Network/ISP owner can act without repeating basic discovery.

## Step 8: Communicate and close

Provide workaround and status cadence during outage.

After restoration, confirm with affected user(s) and record root cause if supplied.

Checkpoint: Closure includes confirmation and known cause.

# 3. Validation and Completion

User can browse and access required app/VPN.

No unexplained packet loss or repeated disconnect during test.

Office/ISP case contains timestamps and diagnostics.

User confirms resolution or workaround.

# 4. Exception and Escalation

Widespread outage: notify network/management owner immediately.

Security/certificate/Conditional Access failure: escalate to relevant owner.

Suspected compromised Wi-Fi/VPN or unusual certificate prompt: stop and escalate to security.

# References

Note: Portal labels can change. Follow the current company policy, approved roles, and tenant-specific configuration if it differs from this general SOP.